Got it — we’ll do this properly, end-to-end, no hand-waving.
Goal recap (tell me if any assumption is wrong):

- 🪟 Windows PC at home = WireGuard server

- 🌐 Clients connect from outside

- 🏠 Clients can:

  - Access LAN devices (router, NAS, printers, other PCs)

  - Use home internet (full tunnel or split tunnel — I’ll show both)

This is totally doable on Windows, just slightly more steps than Linux.

Big Picture (so it all makes sense)

Remote Client

|

|  (encrypted WireGuard tunnel)

|

[Home Router] --- [Windows PC = WG Server] --- LAN devices

|

+--- Internet (NAT)

Your Windows PC will:

- Run WireGuard

- Route packets between:

  - WireGuard tunnel

  - LAN

  - Internet

- NAT traffic so LAN + internet work correctly

Step 0 — Prerequisites

Hardware / network

- Home router you control

- Windows PC that:

  - Is always on (or at least when you want access)

  - Has a static LAN IP (important)

- Admin access on Windows

Software

- Windows 10/11

- WireGuard for Windows

Step 1 — Give the Windows PC a static LAN IP

You must do this either on the router (recommended) or Windows.

Example

Let’s assume:

- Router: 192.168.1.1

- Windows PC (WG server): 192.168.1.50

- LAN subnet: 192.168.1.0/24

Set this in your router’s DHCP reservation page if possible.

Step 2 — Install WireGuard on Windows

- Download: https://www.wireguard.com/install/

- Install normally

- Launch WireGuard (GUI)

Step 3 — Enable IP forwarding in Windows (CRITICAL)

Windows does not route packets by default.

Enable routing

Open PowerShell as Administrator:

Set-ItemProperty `

-Path "HKLM:\SYSTEM\CurrentControlSet\Services\Tcpip\Parameters" `

-Name IPEnableRouter `

-Value 1

Reboot Windows

This is mandatory.

Step 4 — Create WireGuard server tunnel

In WireGuard GUI:

- Click Add Tunnel → Add empty tunnel

- Name it: wg-server

WireGuard will auto-generate keys.

Server Configuration (IMPORTANT)

Paste something like this (adjust IPs if needed):

[Interface]

PrivateKey = <auto-generated>

Address = 10.10.10.1/24

ListenPort = 51820

# Optional but recommended

DNS = 192.168.1.1

What this means

- 10.10.10.0/24 = VPN network

- Server IP = 10.10.10.1

Step 5 — Add Windows Firewall rules

WireGuard usually auto-adds these, but verify:

Allow UDP 51820

- Windows Defender Firewall

- Inbound Rules → New Rule

- Port → UDP → 51820

- Allow

- All profiles

Step 6 — Configure NAT (this is the “Windows pain” part)

We must NAT:

- VPN → LAN

- VPN → Internet

Open PowerShell as Admin

Find your LAN interface name

Get-NetAdapter

You’ll see something like:

- Ethernet

- Wi-Fi

Assume LAN interface = Ethernet

Enable NAT

New-NetNat `

-Name "WireGuardNAT" `

-InternalIPInterfaceAddressPrefix 10.10.10.0/24

That’s it.
Windows will now NAT VPN traffic to LAN + internet.

Step 7 — Router port forwarding

On your home router:

| Setting | Value |
| --- | --- |
| Protocol | UDP |
| External Port | 51820 |
| Internal IP | 192.168.1.50 |
| Internal Port | 51820 |

If your ISP changes your public IP:

- Use Dynamic DNS (DuckDNS, No-IP, etc.)

Step 8 — Add a client (example: laptop or phone)

In WireGuard GUI on server, click:

Edit → Add Peer

Copy the generated peer block.

Client Configuration Example

[Interface]

PrivateKey = <client-private-key>

Address = 10.10.10.2/32

DNS = 192.168.1.1

[Peer]

PublicKey = <server-public-key>

Endpoint = your-public-ip-or-ddns:51820

AllowedIPs = 0.0.0.0/0, ::/0

PersistentKeepalive = 25

What AllowedIPs does

- 0.0.0.0/0 → all internet through home

- If you want LAN only, use:

- AllowedIPs = 10.10.10.0/24, 192.168.1.0/24

Step 9 — Add the client to the server config

On server, add:

[Peer]

PublicKey = <client-public-key>

AllowedIPs = 10.10.10.2/32

Save and Activate the tunnel.

Step 10 — Test (in this order)

From client:

- ✅ Ping VPN server

ping 10.10.10.1

- ✅ Ping router

ping 192.168.1.1

- ✅ Access LAN device

ping 192.168.1.100

- 🌐 Internet test

- Visit https://whatismyipaddress.com
→ should show your home IP

Common Problems & Fixes

❌ No LAN access

- IP forwarding not enabled

- NAT missing

- Wrong AllowedIPs

❌ No internet

- NAT not created

- Client AllowedIPs wrong

- DNS misconfigured

❌ Can’t connect at all

- Router port forwarding wrong

- Firewall blocking UDP

- Wrong public IP / DDNS

Optional (but smart) upgrades

- 🔒 Restrict which LAN IPs clients can access

- 📱 QR codes for mobile clients

- 🔄 Auto-start WireGuard tunnel on Windows boot

- 🌍 Split-tunnel internet only for certain apps

If you want, next I can:

- Draw a network diagram

- Help you lock this down security-wise

- Show PowerShell scripts to auto-recreate NAT after updates

- Optimize for multiple clients

Just say where you want to go next 👍